Quality Management Report

Design of Experiment

26-3Fractional Factorial Design

Instructor: Prof. Jeh-Nan Pan

Group Members:

Nguyen Thi Ngoc Hung (Sarah) – RA6017654

Nguyen Thu Thuy (Aki) – RA8017058

Szu-Chen Ou – R26014103

Wan-Tzu Tsai – R26014064

June, 2013

Contenst

1.	Purpose	3

2.	Data collection	3

3.	Experimental Materials and Process	5

4.	Analysis of Results	6

4.1 Pareto Chart	7

4.2 Normal Probability Plot	7

4.3 Main Effect Plot	8

4.4 Interaction Effect Plot	9

4.4 ANOVA	10

4.5 Regression Model	13

4.6 Predict the Optimal Combination	15

4.7 Confirmation test	16

5.	Conclusion	16

6.	Reference	17

List of Figure

Figure 3.1 Experimental Process	8

Figure 4.1 Pareto chart	8

Figure 4.2 Normal probability effect plot	9

Figure 4.3 Main effect plot	10

Figure 4.4 Interaction effect plot	10

Figure 4.5 Normal probability plot	12

Figure 4.6 Residuals v.s fitted value plot	13

Figure 4.7 Residuals v.s observation value plot	13

Figure 4.8 Normal probability plot	16

Figure 4.9 Residuals v.s fitted value plot	16

Figure 4.10 Residuals v.s observation value plot	17

List of Table

Table 2.1 26-3 fractional factorial design	6

Table 2.2 26-3 fractional factorial design information	6

Table 3.1 Experimental Materials	7

Table 4.1 ANOVA table	11

Table 4.2 Estimated effects and coefficients for response	12

Table 4.3 Regression analysis	15

Table 4.4 Prediction Error	18

- Purpose

We are interested in determining which process variables (factors) affect the launching distance.

Furthermore, we want to determine the region in the important factors that lead to the best possible response (Optimize).

- Data collection

26-3 fractional factorial design

Factor:

Upright arm tension location : medium level and high level

Projector Elevation: medium level and high level

Turn table position: medium level and high level

Pivot arm tension location: medium level and high level

Ball seat position: medium level and high level

Ball type: whiffle and pingpong

Note that we consider medium level as (-) and high level as (+), the reason why we do not consider low level is that the ejection distance will be zero because the ball fall on the catapult.

To test each factor at each level would require a full factorial experiment consisting of 64(26=64) different treatments and that is a large experiment, hence the need to develop some sort of fractional experiment or Taguchi orthogonal array.

We use 26-3 fractional factorial design with 2 replications and the design table is shown below.

Table 2.1 26-3 fractional factorial design

| RunOrder | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | - | + | - | - | + | - |
| 2 | - | - | + | + | - | - |
| 3 | + | - | - | - | - | + |
| 4 | - | + | + | - | - | + |
| 5 | + | - | - | - | - | + |
| 6 | - | - | + | + | - | - |
| 7 | + | - | + | - | + | - |
| 8 | + | + | + | + | + | + |
| 9 | + | + | + | + | + | + |
| 10 | + | - | + | - | + | - |
| 11 | + | + | - | + | - | - |
| 12 | - | - | - | + | + | + |
| 13 | + | + | - | + | - | - |
| 14 | - | + | - | - | + | - |
| 15 | - | - | - | + | + | + |
| 16 | - | + | + | - | - | + |

Table 2.2 26-3 fractional factorial design information

| Factors: 6 | Base Design: 6, 8 | Resolution: III |
| --- | --- | --- |
| Runs: 16 | Replicates: 2 | Fraction: 1/8 |
| Blocks: 1 | Center pts (total): 0 |  |
| * NOTE * Some main effects are confounded with two-way interactions. |  |  |
| Design Generators: D = AB, E = AC, F = BC |  |  |

Alias Structure

I + ABD + ACE + BCF + DEF + ABEF + ACDF + BCDE

A + BD + CE + BEF + CDF + ABCF + ADEF + ABCDE

B + AD + CF + AEF + CDE + ABCE + BDEF + ABCDF

C + AE + BF + ADF + BDE + ABCD + CDEF + ABCEF

D + AB + EF + ACF + BCE + ACDE + BCDF + ABDEF

E + AC + DF + ABF + BCD + ABDE + BCEF + ACDEF

F + BC + DE + ABE + ACD + ABDF + ACEF + BCDEF

AF + BE + CD + ABC + ADE + BDF + CEF + ABCDEF

- Experimental Materials and Process

The materials which we use to do the experiment are shown below:

| Table 3.1 Experimental Materials |  |  |
| --- | --- | --- |
| Materials | source | Number |
| CAT-100 catapult | Laboratory | 1 |
| Tape measure | Laboratory | 1 |
| Scotch tape | 7-11 | 1 |

The experimental process is shown below:

Step1. Generate the fractional factorial design and run order.

Step2. Fix CAT-100 catapult.

Step3. Stick the scotch tape on the table (see Figure 3.1.1).

Step4. Check the run order and the combination of factors to adjust catapult.

Step5. Put the ball on the ball seat position and eject.

Step6. Measure the eject distance.

Figure 3.1 Experimental Process

- Analysis of Results

First, we use Pareto chart and normal probability plot to see what significant effects are. Second, we check main effect plot to find out the best combination and check interaction effect plot to see the significant interaction effects. Third, we use ANOVA and regression model to predict the optimal combination of ejection distance. Finally, we do the confirmation test to see the two models’ ability to prediction.

4.1 Pareto Chart

By figure 4.1, we take α=0.05 and we find that all factors except AF are significant because these factors lie in the right of the red line.

Figure 4.1 Pareto chart

4.2 Normal Probability Plot

By figure 4. 2 (α=0.05). All factors are significant, the result is accordance with the result of Pareto chart (see Figure 4.1).

Figure 4.2 Normal probability effect plot

4.3 Main Effect Plot

Because all main effects are significant, we check figure 4. 3 to see what the best combination is. The best combination is A(1) B(-1) C(-1) D(1) E(1) F(1), which means if we choose Upright Arm Tension Location (A) in high level, Projector Elevation (B) in low level, Turn Table Position (C) in low level, Pivot Arm Tension Location (D) in high level, Ball Seat Position (E) in high level and Ball Type (F) in high level will cause optimal distance.

Figure 4.3 Main effect plot

4.4 Interaction Effect Plot

By the Interaction Plot (figure 4. 4), we can find that except AF interaction effect, the other 2-way interaction effects are significant.

Figure 4.4 Interaction effect plot

4.4 ANOVA

By the Pareto chart and normal probability effect plot, we consider the significant effects which are A, B, C, D, E, F to do ANOVA test.

We confirm the significant effects by the p-value which is less than 0.05.

yijklmn=μ+αi+βj+γk+δl+ηm+θn+εijklmn

The notations means:

yijklmn:ejection distance

αi:effect of Upright Arm Tension Location

βj:effect of Projector Elevation

γk:effect of Turn Table Position

δl:effect of Pivot Arm Tension Location

ηm:effect of Ball Seat Position

θn:effect of Ball Type

εijklmn:the error term

Table 4.1 ANOVA table

| Source | DF | Seq SS | Adj SS | Adj MS | F | P |
| --- | --- | --- | --- | --- | --- | --- |
| Main Effects | 6 | 17113.8 | 17113.8 | 2852.30 | 184.93 | 0.000 |
| Upright Arm Tension Location (A) | 1 | 3896.9 | 3896.9 | 3896.88 | 252.65 | 0.000 |
| Projector Elevation (B) | 1 | 3525.4 | 3525.4 | 3525.39 | 228.57 | 0.000 |
| Turn Table Position (C) | 1 | 2403.5 | 2403.5 | 2403.45 | 155.83 | 0.000 |
| Pivot Arm Tension Location (D) | 1 | 4819.8 | 4819.8 | 4819.83 | 312.49 | 0.000 |
| Ball Seat Position (E) | 1 | 2234.9 | 2234.9 | 2234.93 | 144.90 | 0.000 |
| Ball Type (F) | 1 | 233.3 | 233.3 | 233.33 | 15.13 | 0.006 |
| Residual Error | 9 | 138.8 | 138.8 | 15.42 |  |  |
| Lack of Fit | 1 | 4.7 | 4.7 | 4.73 | 0.28 | 0.610 |
| Pure Error | 8 | 134.1 | 134.1 | 16.76 |  |  |
| Total | 15 | 17252.6 |  |  |  |  |

Table 4.2 Estimated effects and coefficients for response

| Term | Effect | Coef | SE Coef | T | P |
| --- | --- | --- | --- | --- | --- |
| Constant |  | 111.36 | 0.9818 | 113.42 | 0.000 |
| Upright Arm Tension Location (A) | 31.21 | 15.61 | 0.9818 | 15.90 | 0.000 |
| Projector Elevation (B) | -29.69 | -14.84 | 0.9818 | -15.12 | 0.000 |
| Turn Table Position (C) | -24.51 | -12.26 | 0.9818 | -12.48 | 0.000 |
| Pivot Arm Tension Location (D) | 34.71 | 17.36 | 0.9818 | 17.68 | 0.000 |
| Ball Seat Position (E) | 23.64 | 11.82 | 0.9818 | 12.04 | 0.000 |
| Ball Type (F) | 7.64 | 3.82 | 0.9818 | 3.89 | 0.004 |
| S = 3.92733 PRESS = 438.726 R-Sq = 99.20% R-Sq(pred) = 97.46% R-Sq(adj) = 98.66% |  |  |  |  |  |

Next, we should check if the residuals follow four assumptions:

- The residuals are normally distributed.

- The residuals have constant variance.

- The residuals have zero mean.

- The residuals are independence

Figure 4.5 shows that the normality assumption is okay and figure 4.6 shows the residuals are around zero and variance is homogenized. Finally, there is no pattern in figure 4.7, so we conclude that the residuals are random which means they are independent.

Figure 4.5 Normal probability plot

Figure 4.6Residuals v.s fitted value plot

Figure 4.7 Residuals v.s observation value plot

4.5 Regression Model

By the Pareto chart and normal probability effect plot, we consider the significant effects which are A, B, C, D, E, F to predict regression model.

Table 4.3 Regression analysis

| Predictor | Coefficient | SE Coef | T | P |
| --- | --- | --- | --- | --- |
| Constant | 111.356 | 0.982 | 113.42 | 0.000 |
| Upright Arm Tension Location (A) | 15.6063 | 0.9818 | 15.90 | 0.000 |
| Projector Elevation (B) | -14.8438 | 0.9818 | -15.12 | 0.000 |
| Turn Table Position (C) | -12.2563 | 0.9818 | -12.48 | 0.000 |
| Pivot Arm Tension Location (D) | 17.3562 | 0.9818 | 17.68 | 0.000 |
| Ball Seat Position (E) | 11.8188 | 0.9818 | 12.04 | 0.000 |
| Ball Type (F) | 3.8187 | 0.9818 | 3.89 | 0.004 |
| S = 3.92733 R-Sq = 99.2% R-Sq(adj) = 98.7% |  |  |  |  |

By table 4.3, we can express the regression model as follows:

yi=111.356+15.6063xA-14.8438xB-12.2563xC+17.3562xD+11.8188xE+3.8187xF

And the notations mean:

yi：Predicted ejection distance

xA：Upright Arm coding variables {-1, for medium level1,for high level

xB：Projector Elevation coding variables {-1, for medium level1,for high level

xC：Turn Table coding variables {-1, for medium level1,for high level

xD：Pivot Arm coding variables {-1, for medium level1,for high level

xE：Ball Seat coding variables {-1, for medium level1,for high level

xF：Ball Type coding variables {-1, for whiffle         1,for pingpong

After regression analysis, we should check model assumptions:

1. The residuals are normally distributed.

2. The residuals have constant variance.

3. The residuals have zero mean.

4. The residuals are independence.

Figure 4.8 shows that the normality assumption is okay and figure 4.9 shows the residuals are around zero and variance is homogenized. Finally, there is no pattern in figure 4.10, so we conclude that the residuals are random which means they are independent.

Figure 4.8 Normal probability plot

Figure 4.9 Residuals v.s fitted value plot

Figure 4.10 Residuals v.s observation value plot

4.6 Predict the Optimal Combination

By main effect plot (figure 4.3), we know the best combination is A(1) B(-1) C(-1) D(1) E(1) F(1). In this section, we use ANOVA and regression model to predict the optimal response as follows:

- ANOVA prediction model:

yijklmn=111.356+αi+βj+γk+δl+ηm+θn

By the ANOVA model, we can predict the optimal response is:

111.356+15.61+14.84+12.26+17.36+11.82+3.82=187.066

- Regression prediction model：

yi=111.356+15.6063xA-14.8438xB-12.2563xC+17.3562xD+11.8188xE+3.8187xF

We can know that if we choose the ball at high level and take the optimal combination, the result is:

111.356+15.6063+14.8438+12.2563+17.3562+11.8188+3.8187=187.0561

4.7 Confirmation test

Here we use MSE (Mean Square Error) and MAPE (Mean Absolute Percentage Error) to check if both of the prediction models have the ability to prediction.

We do confirmation test by random sampling five observations and five confirmation results are as follows (see table 4.4):

Error=Observations-Prediction valueObservations               MAPE=i=15Error5

Table 4.4 Prediction Error

| Prediction Error |  |  |  |
| --- | --- | --- | --- |
| No | Observations | Regression model error | ANOVA model error |
| 3 | 132.3 | 2.64% | 2.305% |
| 6 | 96 | 4.22% | 4.79% |
| 9 | 132.6 | 0.193% | 0.603% |
| 12 | 157.8 | 1.24% | 1.584% |
| 15 | 152.8 | 1.99% | 1.636% |
|  |  | MAPE=2.0566% | MAPE=2.1836% |

Because all prediction errors are smaller than 15% and MAPEs are both smaller than 10%, we can say both models have the ability to prediction.

- Conclusion

No matter ANOVA or regression model, they both show all main effects are significant which might because of confounding. And in this experiment we use 26-3 fractional factorial design that resolution is only III. Thus, the main effect may not be significant but due to the influential interaction effect cause the wrong conclusion. Therefore, we suggest that using 26-1 or 26-2 factional factorial design would be better.

- Reference

- American National Standards Institute Inc. (2001). For Portable Primary Cells and Batteries With Aqueous Electrolyte - General and Specifications. U.S.

- Chiang, C.-Y. (2002). A study of Gauge repeatability and reproducibility for measurement systems. (Master), National Cheng Kung University, Tainan, Taiwan.

- Montgemery, D. C. (2009). Introduction to Statistical Quality Control. Jefferson City, US: John Wiley & Sons, Inc.

- Montgemery, D. C. (2009). Design and Analysis of Experiments. Jefferson City, US: John Wiley & Sons, Inc.

- 潘浙楠，江巧玉「量測系統重複性與再現性的分析研究」