Quality Management

Term Project Report

Evaluating Gauge Repeatability and Reproducibility for the Weight of One Specific Brand of Golf Balls.

Group 3 Reporters

蔡佩真 Pei-Jen, Tsai R26991062

蘇則維 Tse-Wei, Su R26001134

林  超 Lin Chao   RA7001070

Instructor: Professor Jeh-Nan Pen

Date : June 21, 2012

Chapter 1.	Introduction	3

1.1.	The motivation and background of study	3

1.2.	The goal of study	3

Chapter 2.	Literature review and discussion	4

2.1.	Preface	4

2.2.	Gauge Repeatability and Reproducibility	5

2.3.	ANOVA	6

Chapter 3.	Processing the Experiment	8

3.1.	Important factors analysis:	8

Chapter 4.	Analysis the data of measurement	9

4.1.	Descriptive Statistics	9

4.2.	R Control Chart	9

4.3.	ANOVA analysis with interaction	11

4.4.	The Classical GR&R analysis with interaction	13

4.5.	Discussion the Results	14

4.6.	The Capability of Process	15

Chapter 5.	Conclusion	16

Reference	17

Appendix	18

- Introduction

  - The motivation and background of study

Every golfer in playing golf, no matter what ages he was, always competed who’s driving distance would be the longest. And according to the physics there are two ways to improve it: one is to produce the maximal momentum by the speed of swing and the other is the golf ball weight, diameter, and dimple.

There are many brands of golf ball and the advertisement of every product is focus on its flying distance. According to the USGA the Rules Appendix III

Weight

Appendix III, 2 states that a ball must not weigh more than 1.620 ounces  avoirdupois (45.93 gm). This is one of the simpler tests because all that is required is an accurate scale. There is no minimum weight, thus a ball can be as light as the manufacturer desires.

Size

The ball must have a diameter of not less than 1.680 inches (42.67 mm). The instrument used to measure size is a metal ring gauge. It is important to note that there is no maximum size, the ball can be as large as desired provided it conforms to all other standards.

We want to know the most expensive golf ball in the market and its commercial advertisement shown it has the longer distance. So we use the weight gauge to measure this brand of golf ball whether it was fitted the criteria or not. Also we can estimate if there were errors at measuring.

  - The goal of study

Application of what learn at this semester:

We want to apply the theory what we learn from class to real work. And we want to know through the design of experiment to see whether there was variation or not.  The accuracy and stability of the Gauge and the Operators are adequate:

Use the P/T ratio to detect the stability of the accuracy of operators and gauge.

Detect the variance of products:

Use this method to detect the products if they are conforming or not.

- Literature review and discussion

  - Preface

Determining the capability of the measurement system is an important aspect of many quality and process improvement activities. The measurement system consists of many factors such as gauge, operators, the conditions, different points in time and the skill to use the gauge. There are two important factors to impact the measurement system one is gauge and the other is operator. Firstly, we need to realize two concepts : one is Accuracy and the other is Precision. We use figures to explain these two concepts(fig.1). If the measure is accuracy but not precision is means Gauge Reproducibility and it results from Operator’s error. If it is precision but not accuracy that means Gauge Repeatability and it results from Instrument’s error. Then we use the P/T ratio to estimate the gauge capability.  The others is ANOVA.

Figure 1

A: The gauge is neither accurate nor precise

B: The gauge is accurate but not precise

C: The gauge is not accurate but it is precise

D: The gauge is accurate and precise

  - Gauge Repeatability and Reproducibility

Gauge error : When the inspector uses same gauge to measure a random product several times under the same circumstance, then will get several different results of measurement . This different results, called repeatability, also called Gauge error.

Inspector error : When different inspectors measure a same random product under the same circumstance is called reproducibility. There were different results and this difference called reproducibility, also called inspector error. Therefore, the variance of the total observed measurement as:

σtotal2 = σpart2 + σgauge2

σgauge2 = σrepeatability2 + σreproducibility2

We use the X-R control chart to get data of σrepeatability2 , σreproducibility2

σrepeatability2 =  (R/d2)2

σreproducibility2 = (Rx/d2)2

R = i=1nRi/n

Where n=inspector’s number, R is the range of repeated measurements averaged across parts

Rx = Xmax- Xmin

Where Xmax is the mean of sample average consisting of replicated data obtained by one inspector and is the maximum between inspectors. And Xmin is the minimum between inspectors

P/T ratio = kσ/(USL-LSL) and k = 5.15 and k= 6. k=5.15 corresponds to the limiting value of the number of SD between bounds of a 95% tolerance intervals that contains at least 99% of a normal population. k=6 corresponds to the number of SD between the usual natural tolerance limits of a normal population.

  - ANOVA

If there are a number of randomly selected parts and randomly selected operators, and each operator measures every part n times then the measurements could be represented by the model

Yijk= + Pi + Oj + (PO)ij + εijk

i= 1, 2, …, P; j= 1, 2, …, O; k= 1, 2,…, n

Pi is the effect of parts; Oj is the effect of operators, (PO)ij is the interaction or joint effect of parts and operators; εijk is the random error.

This is a random effect model analysis of variance (ANOVA). It is also sometimes called the standard model for a gauge R & R experiment. We assume that the random variables Pi, Oj, (PO)ij, and εijk, are normally distributed with mean zero and variances given by V(Pi) = σp2, V(Oj) = σo2, V(PO)ij= σpo2 , V(εijk) = σ2

Therefore, the variance of any observation is:

V(Yijk) = σp2 + σo2 + σpo2 + σ2

σp2 , σo2 , σpo2 , σ2 are the variance components.

The procedure involves partitioning the total variability in the measurement into the following component parts:

SStotal =  SSparts + SSoperators + SSPxO + SSerror

SS repents a sum of squares.

Each sum of squares divided by its degrees of freedom to produce mean squares:

MSp  = SSparts/p-1

MSo  = SSoperators/o-1

MSpo = SSPxO/(p-1)(o-1)

MSe   = SSerror/po(n-1)

And Expected mean square by ANOVA as follows :

EMSp=σe2+ kσpo2 + pkσp2
EMSo= σe2 + kσpo2 + nkσo2
E(MSpo)= σe2 + kσpo2
EMSe=σe2

The estimated values of these sources of variation as follow :

σe2 = MSe

σpo2 = (MSpo-MSe)/k

σp2 = (MSp-MSpo)/pk

σo2 = (MSo-MSpo)/nk

Then the repeatability, reproducibility, and the variability of gauge can be calculated through the following :

σrepeatability2 = σe2 = MSE

σreproducibility2=σo2 + σpo2

=(MSo + (n-1)MSpo - nMSE)/nk

σgauge2 = σrepeatability2 + σreproducibility2

= [MSo + (n-1)MSpo + n(k-1)MSE]/nk

We use a practical data from the normal distribution with mean  and variation σ2 and choose these two methods to estimate the total sample variation. Then the biases of estimation can be obtained by comparing the difference between estimated total sample variation and the known variation σ2

- Processing the Experiment

  - Important factors analysis:

Analyze the impossible factors which might cause the different results of measure.

Figure 2

Correctly use the digital scale weight measurement.

We make the rules as follow:

- Same Position.

- Same Method.

- Return the scale to Zero after each measurement,

- Same Device

Design of experiment.

- A specific brand of golf balls

- The weight of this kind of golf balls

- The upper limits is 45.93 gm

- The measure device’s precision is 0.0000 gm

Decide the parameters of measurement

- Objectives: 30 golf balls

- Operators : 3 persons

- Repeat times : 2 times

- Total times of measurements: 30*3*2 = 180 times

Steps of experiment:

- Randomly select the balls

- Assign the operator randomly

- After three operators done then select the next one.

- Repeat the procedure again till 30 balls were done.

- Do the same procedure once again.

- Record the data one by on and input to the Minitab software.

- Analysis the data of measurement

  - Descriptive Statistics

We first show the descriptive statistics to detect the mean and standard deviation of the weight of the Golf Ball, by operator. Then, the total mean and standard deviation were calculated. Because the data of 20 samples was enough so we randomized taking the results of 20 balls for analysis.

| Table.1 Weight |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operator | N | Mean | St.Dev | Min | Max |
| 1 | 40 | 45.731 | .0775 | 45.567 | 45.856 |
| 2 | 40 | 45.731 | .0797 | 45.564 | 45.852 |
| 3 | 40 | 45.725 | .0776 | 45.563 | 45.849 |
| Total | 120 | 45.729 | .0777 | 45.563 | 45.856 |

  - R Control Chart

In statistical quality control, the X and R chart is a type of control chart used to monitor a variable's data when samples are collected at regular intervals from a business or industrial process. When dealing with a quality characteristic that is a continuous variable, it is usually necessary to monitor both the mean value of the quality characteristic and its variability. The process can be monitored by using X control chart to ensure it is in control. The process variability can be monitored by using the R control chart. The X and R charts are the most important and useful on-line statistical tools for monitoring and controlling the process.

When setting up X and R charts, it is better to begin with the R chart, because the control limits on the X chart will depend on the process variability. Unless the process variability is in control, such limits will not have much meaning. Now, we use R control chart to test whether out-of-control conditions are present. The R charts are shown in Figure 3 to Figure 5.

Figure 3

Figure 4

Figure 5

The above control charts were constructed with Minitab. When the 20 units sample range is plotted on the R chart, there is no out-of-control for the weight of the golf balls.

  - ANOVA analysis with interaction

The purpose of the experiment is to check whether the different operators, measurement devices and sample size will influence the results obtained.

| Table.2 Two-way ANOVA table for Operator and Sample |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Source | DF | SS | MS | F | P |
| Operator | 2 | 0.000533 | 0.0002665 | 0.92 | .405 |
| Sample | 19 | 0.614033 | 0.0323175 | 111.19 | <.001*** |
| Interaction | 38 | 0.013685 | 0.0003601 | 1.24 | .225 |
| Error | 60 | 0.017440 | 0.0002907 |  |  |
| Total | 119 | 0.645691 |  |  |  |
| S = 0.01705 R-Sq = 97.30% R-Sq(adj) = 94.64% ***: Significant at α = .05 |  |  |  |  |  |

The ANOVA Table, shows that the weight of Golf Ball has no significant difference between different operators (p-value = .40 > .05), but the sample size is significant (p-value <.001). The interaction term between the Operator and the Sample is not significant (p-value = .225 > .05), so we thought that the material of samples is the factor with much influence on the value of the weight.

Figure 6

The normal probability plot of the residuals and the histogram, shows that the residuals follow the normal distribution and scatter randomly. That means the variance is homogeneous. And the simple linear regression fits this model, so we can use it to predict the weight for different operators and samples.

Figure 7

There was no difference between operators according to the normal probability plot.

Table.3 Variance Decomposition of GRR

Table.4 Variance Decomposition of GRR

  - The Classical GR&R analysis with interaction

Figure 8 GRR analysis

Table. 5 Variance Decomposition of GRR with Interaction

Table.6 Variance Decomposition of GRR

  - Discussion the Results

- Components of Variation

The main effect of measurement variable is Part-to-Part, that is means the main effect of measurement variable is Golf Balls itself.

- R chart by Operators

Although, the measurement variables of all operators are in control, the variance of operator 3 is larger than others.

- X bar chart by Operators

Three operators’ measurement values are out of control of  X chart but in control of R chart, that is the interval of measurement value has no difference. The great variations between Golf Balls lead to this result.

- Weight by Samples

The weight is significant difference between different samples.

- Weight by Operators

The measurement values for three operators are almost equal, and the interval of measurement value has no difference obvious, that is no difference between three operators.

- Operators*Samples Interaction

The means of each sample measured by different operators are almost equal. Therefore, it is no interaction between operators and samples.

- NDC

The numbers of distinct categories of ANOVA and Classical GR&R methods are greater than 5 so the experiment was good.

  - The Capability of Process

CPu= USL-LSL6σ

CPu= USL-X3σ

CPl= X-LSL3σ

CPK =min⁡{X-LSL3σ,USL-X3σ}

X = 45.7293, S= .0777, USL=45.9300, Cp= .86, Cpu = 0.86, Cpk = .86

So the capability of process is good.

- Conclusion

An adequate measurement system is very important in quality improvement in industry. We move the learning in class to do the experiment to analyze the gauge repeatability and reproducibility to determine whether the precision and accuracy of measurement systems are acceptable. A reasonable acceptance range of P/T ratio has also been derived but our study no lower limit so we can’t use this method to estimate our results. But the number of distinct categories is another choice for estimate so our NDCs both are greater than 5, so our experiment showed very good. Although we estimated 30 samples but as we know 20 samples are enough so we selected 20 samples to run our Minitab software. The accuracy of two methods for estimate the GR&R are compared. And ANOVA method is the most accurate one since it includes the variation of interaction between product and inspector and it can be done with Minitab statistical software. ANOVA has smaller bias than Classical GR&R but if the numbers of measurement are very large there were no differences but it cost too much.

Reference

1. JEH-NAN PAN. Evaluating the Gauge Repeatability and Reproducibility for Different Industries. Quality & Quantity (2006) 40:499–518

2. Douglas C. Montgomery. Statistical Quality Control a modern introduction 6e

3. 陳玟伶. 鈕扣的量測變異分析

| Appendix |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operator 1 |  |  |  | Operator 2 |  |  |  | Operator 3 |  |  |  |
| Repeat 1 |  | Repeat 2 |  | Repeat 1 |  | Repeat 2 |  | Repeat 1 |  | Repeat 2 |  |
| Number | weight | Number | weight | Number | weight | Number | weight | Number | weight | Number | weight |
| 1 | 45.6837 | 1 | 45.6850 | 1 | 45.6801 | 1 | 45.6805 | 1 | 45.6762 | 1 | 45.6792 |
| 2 | 45.7324 | 2 | 45.7334 | 2 | 45.7295 | 2 | 45.8004 | 2 | 45.7990 | 2 | 45.7991 |
| 3 | 45.6417 | 3 | 45.6433 | 3 | 45.6366 | 3 | 45.6388 | 3 | 45.6324 | 3 | 45.6349 |
| 4 | 45.8263 | 4 | 45.8269 | 4 | 45.8247 | 4 | 45.8250 | 4 | 45.8212 | 4 | 45.8224 |
| 5 | 45.7503 | 5 | 45.7508 | 5 | 45.7460 | 5 | 45.7470 | 5 | 45.7432 | 5 | 45.7447 |
| 6 | 45.7505 | 6 | 45.7510 | 6 | 45.7479 | 6 | 45.7483 | 6 | 45.7431 | 6 | 45.7434 |
| 7 | 45.7324 | 7 | 45.7332 | 7 | 45.7291 | 7 | 45.7304 | 7 | 45.7268 | 7 | 45.7284 |
| 8 | 45.7020 | 8 | 45.7021 | 8 | 45.6984 | 8 | 45.6985 | 8 | 45.6955 | 8 | 45.6958 |
| 9 | 45.7393 | 9 | 45.7394 | 9 | 45.7342 | 9 | 45.7355 | 9 | 45.7324 | 9 | 45.7338 |
| 10 | 45.6213 | 10 | 45.6216 | 10 | 45.6187 | 10 | 45.6194 | 10 | 45.6163 | 10 | 45.6165 |
| 11 | 45.6559 | 11 | 45.6566 | 11 | 45.6518 | 11 | 45.6539 | 11 | 45.6502 | 11 | 45.6515 |
| 12 | 45.5674 | 12 | 45.5685 | 12 | 45.5643 | 12 | 45.5648 | 12 | 45.5628 | 12 | 45.5638 |
| 13 | 45.7008 | 13 | 45.7934 | 13 | 45.7890 | 13 | 45.7912 | 13 | 45.7871 | 13 | 45.7875 |
| 14 | 45.7655 | 14 | 45.7663 | 14 | 45.7631 | 14 | 45.7638 | 14 | 45.7605 | 14 | 45.7611 |
| 15 | 45.6271 | 15 | 45.6283 | 15 | 45.6241 | 15 | 45.6243 | 15 | 45.6205 | 15 | 45.6208 |
| 16 | 45.7941 | 16 | 45.7945 | 16 | 45.7894 | 16 | 45.7907 | 16 | 45.7876 | 16 | 45.7881 |
| 17 | 45.7098 | 17 | 45.7104 | 17 | 45.7066 | 17 | 45.8522 | 17 | 45.7050 | 17 | 45.7058 |
| 18 | 45.6741 | 18 | 45.6746 | 18 | 45.6720 | 18 | 45.6722 | 18 | 45.6683 | 18 | 45.6705 |
| 19 | 45.7597 | 19 | 45.7602 | 19 | 45.7581 | 19 | 45.7585 | 19 | 45.7542 | 19 | 45.7558 |
| 20 | 45.8544 | 20 | 45.8561 | 20 | 45.8508 | 20 | 45.8519 | 20 | 45.8469 | 20 | 45.8492 |
| 21 | 45.8128 | 21 | 45.8129 | 21 | 45.8093 | 21 | 45.8099 | 21 | 45.8069 | 21 | 45.8083 |
| 22 | 45.7632 | 22 | 45.7635 | 22 | 45.7575 | 22 | 45.7606 | 22 | 45.7574 | 22 | 45.7583 |
| 23 | 45.8236 | 23 | 45.8247 | 23 | 45.8204 | 23 | 45.8216 | 23 | 45.8172 | 23 | 45.8174 |
| 24 | 45.7429 | 24 | 45.7431 | 24 | 45.8101 | 24 | 45.8102 | 24 | 45.7378 | 24 | 45.7382 |
| 25 | 45.7363 | 25 | 45.7369 | 25 | 45.7324 | 25 | 45.7340 | 25 | 45.7306 | 25 | 45.7310 |
| 26 | 45.7617 | 26 | 45.7619 | 26 | 45.7576 | 26 | 45.7595 | 26 | 45.7551 | 26 | 45.7557 |
| 27 | 45.8234 | 27 | 45.8243 | 27 | 45.8198 | 27 | 45.8208 | 27 | 45.8175 | 27 | 45.8181 |
| 28 | 45.7533 | 28 | 45.7537 | 28 | 45.7332 | 28 | 45.7492 | 28 | 45.7461 | 28 | 45.7467 |
| 29 | 45.8039 | 29 | 45.8043 | 29 | 45.7294 | 29 | 45.8016 | 29 | 45.7266 | 29 | 45.7273 |
| 30 | 45.8271 | 30 | 45.8278 | 30 | 45.8241 | 30 | 45.8242 | 30 | 45.8222 | 30 | 45.8233 |