Quality Management Final Report

The Measurement System Capability Analysis for Extra sugarfree gum

Prepared for Professor Pan

Quality Management Spring 2012

Instructor: Professor Pan, J.N.

潘浙楠教授

Prepared by:

R26001142 林振宇

R26004077 黃冠禎

RA7991152 吳文仁

Contents

ABSTRACT	2

1	PROBLEM FORMULATION	2

1.1	Motivation	2

1.2	Purpose of Research	2

2	RESEARCH PROCEDURES AND METHOD	2

2.1	The project flow	2

2.2	Cause and Effect Analysis	4

2.3	Sub-Process for each of Measuring Weight:	6

3	SAMPLE COLLECTION	7

3.1	Sample Description	7

3.2	Lab. Environment and Measuring Equipment	7

3.3	Measurement Process	8

4	Data Analysis	8

4.1	Descriptive Statistics	8

4.2	Normality Tests	9

4.3	R Control Chart	11

4.4	ANOVA Analysis	13

4.5	Gauge Repeatability and Reproducibility	14

4.6	Process Capability Cpl	16

5	CONCLUSION	18

REFERENCES	18

- ABSTRACT

This study is to verify the integrity of the declared chewing gum weight, and further explore the process capability from taking random samples from market. The measurement results reveal that the measurement system is reliable, the operators’ performance is good, and no samples were below the declared specification, consumers’ benefit is secured, and the process capability is to be improved.

PROBLEM FORMULATION

Motivation

In the old days, the specification of snack food was defined by numbers of pices and weight per bag, like 12 pieces and 30 gram / net weight. The over-specification products, either underweighted due to insufficient numbers of pieces or over-weighted resulted from extra pieces or unexpected contamination, can be sorted by on-line scale, so called inspection quality. At field survey, our team find that the current chewing gum makers declare the weight by ‘unit’ weight and net total in one bag. One unit consists of two pieces randomly, no upper or lower limit so the declared weight is assumed as minimum weight. This means that the makers can only secure their integrity by statistical quality control, i.e. manufacturing quality. The question is that does consumers’ benefit be secured by the declared weight, not only total but also by piece? Can the maker manage the production capability?

Purpose of Research

The purpose of the project is to measure the weight of chewing gum, using statistical technique to verify the compliance of the specification, and estimate its production capability. Meanwhile, the team will take this chance to learn from  precision weight measuring process as a QC engineer.

RESEARCH PROCEDURES AND METHOD

The project flow

The project starts from making a sampling plan and ended by discussion of the findings, see Figure 2.1. As the previous study (Pan, 2004), we decide to take randomly five units from each bag, total 20 units of samples to satisfy the confidence level. After examining the purchased samples and visited the lab., the team worked out a cause analysis chart in order to define the control variables.

The experiment started after the lab. was in a stable and homogeneous environmental condition. The team randomly sampled five units from each of the four bags, confirmed that every piece is intact, lay down unit by unit on a matrix paper. During the waiting time of letting the samples cooling down to the lab. temperature, we calibrated the weight scale by 200 grams weight block, and practice the handling process to prevent errors. Since any marking on the samples will result in change of weight, we marked numbers from one to twenty to identify the samples by unit. Three operators participate this experiment, taking no. 1 to no. 3. The random measuring sequential schedule was created by Minitab. When operator 1 was measuring, he spoke out the reading, operator 2 repeated the reading and key-in the Minitab file, and operator 3 double check the input data are correct. Three operators rotated twice to get total 120 observations.

When the operators completed the measurement, the team used Minitab to do gauge repeatability and reproductability analysis, then discuss the results.

Figure：2.1 The Overall Process

Cause and Effect Analysis

We apply six categories – people, material, method, measurement, environment to work out a cause and effect diagram and define the necessary control variables. The diagram is as Figure 2.2, and the solutions are listed in Table 2.2.

Figure 2.2: Cause and Effect Diagram

Table 2.2 
The Solutions of Cause and Effect Analysls

| Category | Effect | Solution | Evaluation |
| --- | --- | --- | --- |
| Person | Skill | Training by practicing, keep a steady pase | Good |
|  | Touching (sweat, scratching) | Using bamboo chops to deliver the samples | Better tools is expected |
|  | Dropping | Using a plastic cover to prevent dropping down to table | Fair, drop to the cover still result in loss of weight |
| Machine | Start from zero | Calibration before experiment, return to zero at every measurement before open the glass door | Good |
|  | Unstable voltage | UPS is preferable | No resource |
|  | Time variation | Warm up the machine by five minutes; try minimum time to reduce potential variance | Good |
| Material | Oxidation | Try to complete at shortest time to minimize oxidation | Good |
| Method | Position on the scale (unequal reading) | The measuring of the scale is from sensitive piezoelectric materials. Always locate the samples on the center of the scale holder and piece-to-piece to obtain evenly distributed load on scale | Good |
|  | Identification | No marking on the samples so use matrix paper with numbers to identify the samples | Good |
| Measurement | Reading | The first operator makes sure cite the reading when no changing, and double check | Good |
|  | Input | The first operator speaks out the reading, and input by the second operator by repeating | Good |
|  | Communication | The third operator double check by hand-writing note with the second operator at every reading | Good |
| Environ- -ment | Humidity & temp | Stable air conditioning started one hour before the experiment | Good, but better to have meters to verify the conditions |
|  | Air flow & dust | The lab. has two doors, the scale has a glass box with sliding door to minimize the air flow and dust. One plastic holder was placed on the scale to prevent contamination | Good |
|  | Vibration | The scale sits on a rock platform | Good |

Sub-Process for each of Measuring Weight:

In order to obtain accurate and precise measurement, the team define a detailed measuring process illustrated in Figure 2.2. In addition to the basic calibration of the weight scale, the operators practiced handling process, including moving the samples from the matrix paper to the scale in a route, locate the samples on the center of the scale holder in a gentle way, breathing smoothly and avoid any saliva, sweat, and tear if any drop to the samples. Meanwhile, we try to keep a calm and energized mood can ensure an efficient and fluent flow so no visitor is allowed to access the lab. during the experiment except Professor Pan. Always returned to zero is a simple and important step.

Figure 2.3: Sub-Process for each of Measuring Weight

SAMPLE COLLECTION

Sample Description

We went to a convenient store, randomly picking up four bags of one popular brand of chewing gum, each bag contains 13 units, and each unit is 2 pieces. The price is NTD 39 per bag. The maker declares that each unit’s weight is 2.8 gram, so we use it as the lower specification limit (LSL) of the samples.

| Figure 3.1：Extra sugarfree gum, left-general view | Figure 3.2: The product’s specification |
| --- | --- |

Lab. Environment and Measuring Equipment

| The Lab. is located at the fourth floor of the Department of the Statistics. Air conditioning capacity can maintain a steady temperature and humidity. Illumination is sufficient. The weight scale sits on a rock platform and protected by a glass shielding box, see Figure 3.3. The particulars of the weight scale is: - Maker & model: RADWAG, AS220/C/1 - Capacity: max. 220 g, d= 0.1 mg | Figure 3.3：Digital weight scale |
| --- | --- |

Measurement Process

The team uses Minitab (version 15)　installed in a private laptop computer to do data analysis. To prevent contamination, a small plastic cover as a holder with dia75.26 mm, 4.8483g, is placed on the weight scale to sustain the object on weight scale. Considering the weight of the holder is much smaller than the capacity of the scale is 220 g, it is concluded that it will not affect the experiment results. Weight scale is returned to zero after the holder was placed on the scale, so the initial reading is always 0.0000 g.

To minimize the contamination by touching the samples, the operator uses bamboo chops to deliver the samples between the matrix paper and the scale. The chops can be soft and not to hurt the samples, but it is round so it is not easy to hold the samples well. It took time of the team to practice it, and used another plastic cover with dia. 88.16mm, 8.7498g, to prevent dropping at handling.

| Figure 3.4: Measurement Process |  |
| --- | --- |

Data Analysis

Descriptive Statistics

Twenty samples from Extra sugarfree gum were weighted by three operators. We first show the descriptive statistics to detect the mean and standard deviation of the observations by operator, then the total mean and standard deviation were calculated.

| Table：4.1 Descriptive statistics |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operator | N | Mean | St.Dev | Min | Max |
| 1 | 40 | 2.894188 | 0.045829 | 2.8117 | 2.9738 |
| 2 | 40 | 2.894133 | 0.045809 | 2.8115 | 2.9734 |
| 3 | 40 | 2.894073 | 0.045802 | 2.8115 | 2.9735 |

| Table：4.2 Summary statistic |  |  |  |  |
| --- | --- | --- | --- | --- |
| N | Mean | Std | Operators | Parts |
| 120 | 2.8941 | 0.044841 | 3 | 20 |

Normality Tests

When we use the X-R chart and process capability to analyze the data, we all assume that the data are under normal distribution. Before analyzing the observations, we do the normality test. A normal quantile-quantile plot and Shapiro-Wilk test will demonstrate the obserations are normally distributed.

Shapiro-Wilk test：

H0：the weight of the samples from the sugarless chewing gum is normally distributed

H1：the weight of samples from sugarless gum data is not normally distributed

The data are the weight of sugarless gum, which every operator measure twice. We calculate the data mean, and generate normal quantile-quantile plot and Shapiro-Wilk test with different operators' data.

First operator

Shapiro-Wilk normality test：W = 0.9628, p-value = 0.6009

Figure 4.1: First operator Normal quantile-quantile plot

The p-value of Shapiro-Wilk test from first operator data is 0.6009, less than Alpha equal to 0.05, we do not reject H0. The points of data drawn on the Q-Q plot(Figure 4.1), present a straight line. We know the observations are normally distributed, so we can use the X-R chart and process capability to analyze the data.

Second operator

Shapiro-Wilk normality test：W = 0.9625, p-value = 0.595

Figure 4.2: Second operator Normal quantile-quantile plot

The second operator’s p-value of Shapiro-Wilk test is 0.595 and less tha Alpha equal to 0.05, we do not reject H0. The points of data, which drawn on the Q-Q plot(Figure4.2), present a straight line. We know the observations are normally distributed, so we can use the X-R chart and process capability to analyze the data.

Third operator

Shapiro-Wilk normality test：W = 0.9626, p-value = 0.5969

Figure 4.3: Third operator Normal quantile-quantile plot

The third operator’s p-value of Shapiro-Wilk test, which is 0.5969 less than Alpha equal to 0.05, we do not reject H0. The points of data, which drawn on the Q-Q plot(Figure5.3), present a straight line. We know the observations are normally distributed, so we can use the X-R chart and process capability to analyze the data.

R Control Chart

In statistical quality control, the X and R chart is a type of control chart used to monitor a variable's data when samples are collected at regular intervals from a business or industrial process. When dealing with a quality characteristic that is a continuous variable, it is usually necessary to monitor both the mean value of the quality characteristic and its variability. The process mean can be monitored by using X control chart to ensure the process is in control. The process variability can be monitored by using the with the R control chart. The X and R charts are among the most important and useful on-line statistical process monitoring and control techniques.

When setting up X and R charts, it is better to begin with the R chart. because the control limits on the X chart will depend on the process variability. Unless the process variability is in control, such limits will not have much meaning. Now, we use R control chart to test whether out-of-control conditions are present. The R charts are shown in Figure5.4-5.6.

Figure 4.4: First operator R chart

Figure : 4.5 Second operator R chart

Figure 4.6 Third operator R chart

The control charts of Figure 4.4-4.6 show no presence of an out-of-control condition. Finding three operators' measurement are all in control, so we didn't delete any point.

ANOVA Analysis

In this experiment, we have three different operators measuring the weight twice. The purpose of the ANOVA analysis is to check whether the different operators, measurement devices and sample size will influence the results obtained.

(1) ANOVA(with interaction)

| Table：4.3 ANOVA(with interaction) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Source | DF | SS | MS | F | P |
| Parts | 19 | 0.239268 | 0.0125931 | 836602 | <0.001 |
| Operators | 2 | <0.000001 | 0.0000001 | 6.53 | 0.003 |
| Parts * Operators | 38 | 0.000001 | <0.000001 | 0.74 | 0.834 |
| Error | 60 | 0.000001 | <0.000001 |  |  |
| Total | 119 | 0.23927 |  |  |  |
| S = 0.0001423 R-Sq = 100.00% R-Sq(adj) = 100.00% |  |  |  |  |  |

According to the ANOVA Table 4.3, the observations of operators are significant (p-value=0.003<0.05), and the sample (parts) shows to be significant (p-value=<0.001). And the interaction term between Operator and Sample size is not significant (p-value=0.834>0.05), so we conclude there is no operators influence on the observations.

Figure 4.7: The residuals plot

Look at Figure 4.7, the normal probability plot of the residuals and the histogram, shows that the residuals follow the normal distribution and scatter randomly. That means that the variance is homogenized. Therefore, the simple linear regression fits this model, and we can use it to predict the weight for different operators and samples.

Gauge Repeatability and Reproducibility

Figure 4.8: The GRR plot from XBar/R Method

- Components of Variation

According to Figure 4.8, the main effect of measurement variable is Part-to-Part and Repeatability, that is means the main effect of measurement variable is auto-deviation of mercury batteries and operators.

- R chart by operator

The measurement variable of operators are all in control.

- X bar chart by operator

Three operator’s measurement values are out of control of  X chart but in control of R chart, that is the interval of measurement value has no difference obvious. There is great variation between sugarfree gum itself lead to this result.

- Weight by sample

The weight is significant difference between different samples.

- Weight by operator

The measurement values for three operators are almost equal, and the interval of measurement value has no remarkable difference, that is no difference between three operators.

- Operator *Sample Interaction

According to Figure 4.8, the means of each sample measured by different operators are almost equal. Therefore, it is no interaction between operators and samples.

| Table 4.6: GRR with XBar/R Method |  |  |  |  |
| --- | --- | --- | --- | --- |
| Source | StdDev (SD) | Study Var (6*SD) | %Study Var (%SV) | %Tolerance (SV/Toler) |
| Total Gage R&R | 0.0001511 | 0.000907 | 0.35 | 0.48 |
| Repeatability | 0.0001404 | 0.000842 | 0.32 | 0.45 |
| Reproducibility | 0.0000560 | 0.000336 | 0.13 | 0.18 |
| Part-To-Part | 0.0433423 | 0.260054 | 100 | 138.13 |
| Total Variation | 0.0433425 | 0.260055 | 100 | 138.13 |

The part-to-variation, which equal to total variation is 0433425. The total Gage R&R is 0.0001511, which have 0.35% from total variance. That mean people measurement error is very low, and we can calculate SNR to support the conclusion. The standard deviation of total GRR is 0.0001511 and standard deviation of part-to-part is 0.0433423. The SNR function is shown below:

SNR=sd(part-to-part)/sd(total GRR)*1.41=0.0433423/0.0001511*1.41=404.4516

The SNR value is 404.4516 which higher than 5, so we conclude that the  measurement system is appropriate. According to Tables 4.6, we can find the most of the variation which is from the sample.

Process Capability Cpl

Extra sugarfree gum provides the lower specification limit of weight for their product. The process capability is a measurable property of a process to the specification, expressed as a process capability index, Cpl. The process capability ratio is a measure of how a process to manufacture a product meets the customer’s specifications. If the Cpl value is higher than 1.33, it means that the process is good. We calculate the Cpl value with mean=2.8941 and standard deviation=0.044841, which the Cpl value function is shown below:

CPl = (Xbar-LSL)/3σ = (2.8941-2.8)/(3*0.0433423) =0.7236964

The Cpl value is 0.7236964 which lower than 1.33, so we confirm the process capability is not good (grade C). We suggest to improve the process by suitable means.

The manufacturing process of chewing gum is : Material feeding >blending >milling >metal and contamination checking>cooling >cutting >sugar coating>edge cutting>color coating >packaging., illustrated in Figure 4.9. Nowaday it is a highly automatically production process, and using alumi-plastic composited. foil bag to pack is much better than the traditional paper and plastic film bag to resist humidity , oxidation, and ultra violet (UV). As previous study using Taguchi method to improvement the chewing gum performance, the nozzle size of the sugar coating spraying, the angle of nozzle, and the sugar feeding rate are three main factors influence the weight and appearance of the chewing gum. Our team is wondering the air contents of each piece in cooling stage may be another factor but not literature can validate the idea. However, since the total material loss rate is about 5% only, and the low cost and mass production nature of the chewing gum, the manufacturers may prefer to use a higher target weight to prevent under specification claim. But over-weighted chewing gum does not mean the consumers’ benefit increased, but contamination risk exists.

Figure 4.9: The chewing gum manufacturing process

CONCLUSION

The objective is to explore the integrity of the chewing gum about the weight. Since all measured weight of the samples are above the declared lower specification, so consumers’ benefit is secured. The second objective is to measure the maker’s process capability. The results are On-the-way and needs improvement. From previous study, sugar and color coating and cooling processes may be improved. The nozzle width, spraying angle, and feeding rate of coating are three main effects of the weight of the chewing gum. However, since the material loss rate is about 5% only, and to the low cost and mass production nature of the chewing gum, the manufacturer may prefer to use a higher target weight to prevent under specification claim.

The limitation of this project is samples were taken from cross-sectional populations with limited amount. A regular measurement on-line to monitor potential shift is a must.

Future improvement: It was observed some tiny breakdown of the objects on the plastic holder. From Figure 4.7 it can be observed that the second run data were at a pattern generally lower than the first run. The part-to-part deviation may be resulted from the reducing weight. The variance of the operators remains low and no accumulated errors in the data due to returned to zero before any measurement.

- REFERENCES

- Montgomery, D. C. (2009). Statistical quality control (sixth ed.). U.S.A: John Wiley & Sons, Inc.

- Pan, Jeh-Nan, Chiang, (2002), C.Y., Measurement system assessment by experiment study

- Pan, Jeh-Nan (2004), Determination of the optimal allocation of parameters for gague repeatability and reproducibility study, International Journal of Quality & Reliability Management, 21(6), 672-682

- Science Technology Japan, Video of chewing gum production process, http://v.youku.com/v_show/id_XMjg1MTc3NjIw.html

- WRIGLEY chewing gum production flow:　http://www.wrigley.com.tw/T3001ShowUploadFile?y_SketchName=Sketch1-2_Hi178&y_HrefId=140897&y_KindId=141510潘浙楠, & 李文瑞. (2003). 品質管理-理論與實務, 第2版.台北: 華泰書局.

- 何智遠,陳彥妤,魏盛平(2010). 口香糖之品質改善與製程分析.2010機光電技術與應用研討會,台北.

- 高雄國稅局(2010),食品加工業原物料耗用通常水準