Quality Control Final Report

Find out the Best Combination for the CAT-100 Catapult by Fractional Factorial Design

Instructor: Professor Pan, J. N./ Phd.

Quality Management Spring 2012

Prepared by:

R26001142 林振宇

R26004077 黃冠禎

RA7991152 吳文仁

Contents

ABSTRACT	2

1	PROBLEM FORMULATION	2

1.1	Motivation	2

1.2	Purpose of Research	2

2	SAMPLE COLLECTION	2

2.1	Factor Consider	2

2.2	Experimental Materials and Process	3

2.3	Experimental Data	4

3	Analysis of Results	5

3.1	Pareto Chart	5

3.2	Normal Probability Plot	5

3.3	Main Effect Plot and Interaction Plot	6

3.4	ANOVA analysis	7

3.5	ANOVA model	8

3.6	Regression analysis	10

4	CONCLUSION	10

REFERENCES	11

- ABSTRACT

The CAT-100 catapult just like a trebuchet, which is used for sieging the castle, and it has six factors.  By applying the fractional factorial design of experiment, our team finds out the main factors to influence on the throwing distance and the best combination of main factors.

PROBLEM FORMULATION

Motivation

When we were young, we read the story about "Romance of Three Kingdom". The story has the trebuchet used for sieging the castle. We are very interested in shooting distance of trebuchet, so we can know the best combination stimulated by the CAT-100 catapult. This experiment can satisfy our curiosity, and we can learn experiment design.

Purpose of Research

The CAT-100 catapult has six different factors, we choose five of them to do experiment. The five factors are Upright Arm Tension Location, Turn Table Position, Pivot Arm Tension Location, Ball Seat Position and Ball Type. We use the fractional factorial design (25-1) to find out the main factor (effect).

SAMPLE COLLECTION

Factor Consider

We use 25-1 fractional factorial to design an efficient experiment by Minitab software to produce two times. We deleted one factor projector elevation. We randomly picked up the orders to produce total 32 observations.

| Table：2.1 Factors and Their Levels |  |  |
| --- | --- | --- |
| Factor | Factor Name | Level |
| A | Upright Arm Tension Location | 1:Low level,3:High level |
| C | Turn Table Position | 1:Low level,3:High level |
| D | Pivot Arm Tension Location | 1:Low level,3:High level |
| E | Ball Seat Position | 1:Low level,3:High level |
| F | Ball Type | 1:Foam(yellow),3:PingPong(orange) |

Figure 2.1 CAT-100 catapult

Experimental Materials and Process

We use the below materials to make the experiment:

| Table：2.2 Experimental Materials |  |  |
| --- | --- | --- |
| Materials | source | Number |
| CAT-100 catapult | Laboratory | 1 |
| Tape measure | Laboratory | 1 |
| Newspaper | 7-11 | 1 |
| Flour | 7-11 | 1 |

The following is experimental process:

- Use the Minitab to have experimental number

- Put the CAT-100 catapult fixed

- Paper adhesive to the floor

- On the newspaper covered with flour

- Put the ball on the Ball Seat Position

- Measure the ball in flour placement

Experimental Data

| Table 2.3 Factors and Their Identification |  |
| --- | --- |
| Factor | Factor Name |  |
| A | Upright Arm Tension Location |  |
| B | Turn Table Position |  |
| C | Pivot Arm Tension Location |  |
| D | Ball Seat Position |  |
| E | Ball Type |  |

| Table 2.4 Experimental Data |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | C | D | E | obs | obs |
| 1 | 1 | 1 | 1 | 1 | 72 | 81 |
| -1 | 1 | 1 | 1 | -1 | 51 | 49 |
| 1 | -1 | 1 | 1 | -1 | 151 | 151 |
| -1 | -1 | 1 | 1 | 1 | 100 | 101 |
| 1 | 1 | -1 | 1 | -1 | 30 | 31 |
| -1 | 1 | -1 | 1 | 1 | 0 | 0 |
| 1 | -1 | -1 | 1 | 1 | 71 | 77 |
| -1 | -1 | -1 | 1 | -1 | 0 | 0 |
| 1 | 1 | 1 | -1 | -1 | 48 | 54 |
| -1 | 1 | 1 | -1 | 1 | 28 | 28 |
| 1 | -1 | 1 | -1 | 1 | 91 | 93 |
| -1 | -1 | 1 | -1 | -1 | 55 | 59 |
| 1 | 1 | -1 | -1 | 1 | 0 | 0 |
| -1 | 1 | -1 | -1 | -1 | 0 | 0 |
| 1 | -1 | -1 | -1 | -1 | 42 | 41 |
| -1 | -1 | -1 | -1 | 1 | 0 | 0 |
| -1: Low level,1:High level |  |  |  |  |  |  |

Analysis of Results

Pareto Chart

Refer to Figure 3.1, under Alpha=0.05, the factors are significant which cross the red line. We can find those significant factors are C, A, B, D, AB, BC, CD, AD, BD, AE, DE and BE.

Figure 3.1 Pareto Chart

Normal Probability Plot

See Figure 3.2, under Alpha=0.05, we can find the significant factors C, A, B, D, AB, BC, CD, AD, BD, AE, DE and BE, don't attach to the straight line.

Figure 3.2 Normal Probability Plot

Main Effect Plot and Interaction Plot

By Figure 3.3, we can find the main effects, except factor E, have high different for their mean. The best combination is A(1)B(-1)C(1)D(1), which mean A in high level, B in low level, C in high level and D in high level. We can know Upright Arm Tension Location in high level, Turn Table Position in low level, Pivot Arm Tension Location in high level and Ball Seat Position in high level.

Figure 3.3 Main Effect Plot

From Figure 3.4, we can find the Second-order interaction effects, except AC and CE, have signification.

Figure 3.4 Interaction Plot

ANOVA analysis

Base on the above reasons of effect, we establish the model:

Yijklm＝μ＋Ai＋Bj＋Ck＋Dl＋Em＋ABij＋BCjk＋CDkl＋ADil＋BDjl＋AEim＋DElm＋BEjm

i=j=k=l=m=1,2

| Table 3.1 ANOVA analysis (complete) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source | DF | Seq SS | Adj SS | Adj MS | F | P |
| A | 1 | 9870.1 | 9870.1 | 9870.1 | 1370.59 | <0.001 |
| B | 1 | 9800 | 9800 | 9800 | 1360.85 | <0.001 |
| C | 1 | 26450 | 26450 | 26450 | 3672.9 | <0.001 |
| D | 1 | 5671.1 | 5671.1 | 5671.1 | 787.5 | <0.001 |
| E | 1 | 12.5 | 12.5 | 12.5 | 1.74 | 0.204 |
| A*B | 1 | 1830.1 | 1830.1 | 1830.1 | 254.14 | <0.001 |
| B*C | 1 | 1512.5 | 1512.5 | 1512.5 | 210.03 | <0.001 |
| C*D | 1 | 946.1 | 946.1 | 946.1 | 131.38 | <0.001 |
| A*D | 1 | 840.5 | 840.5 | 840.5 | 116.71 | <0.001 |
| B*D | 1 | 406.1 | 406.1 | 406.1 | 56.4 | <0.001 |
| A*E | 1 | 351.1 | 351.1 | 351.1 | 48.76 | <0.001 |
| D*E | 1 | 300.1 | 300.1 | 300.1 | 41.68 | <0.001 |
| B*E | 1 | 242 | 242 | 242 | 33.6 | <0.001 |
| Error | 18 | 129.6 | 129.6 | 7.2 |  |  |
| Total | 31 | 58362 |  |  |  |  |
| S = 2.68354 R-Sq = 99.78% R-Sq(adj) = 99.62% |  |  |  |  |  |  |

By the table 3.1, we find R-Sq(adj) = 99.62%, which means the model is very good. But we find the factor E is not significant in Table 3.1, we deleted such factor and make model again. When we delete the main factor E, we also have to delete the interaction effects about E. So we have total removal factor E, AE, DE, and BE with a simplified model

Yijkl＝μ＋Ai＋Bj＋Ck＋Dl＋ABij＋BCjk＋CDkl＋ADil＋BDjl  ;  i=j=k=l=1,2

| Table 3.2 ANOVA analysis (simplified) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source | DF | Seq SS | Adj SS | Adj MS | F | P |
| A | 1 | 9870.1 | 9870.1 | 9870.1 | 209.72 | <0.001 |
| B | 1 | 9800 | 9800 | 9800 | 208.23 | <0.001 |
| C | 1 | 26450 | 26450 | 26450 | 562.02 | <0.001 |
| D | 1 | 5671.1 | 5671.1 | 5671.1 | 120.5 | <0.001 |
| A*B | 1 | 1830.1 | 1830.1 | 1830.1 | 38.89 | <0.001 |
| B*C | 1 | 1512.5 | 1512.5 | 1512.5 | 32.14 | <0.001 |
| C*D | 1 | 946.1 | 946.1 | 946.1 | 20.1 | <0.001 |
| A*D | 1 | 840.5 | 840.5 | 840.5 | 17.86 | <0.001 |
| B*D | 1 | 406.1 | 406.1 | 406.1 | 8.63 | 0.008 |
| Error | 22 | 1035.4 | 1035.4 | 47.1 |  |  |
| Total | 31 | 58362 |  |  |  |  |
| S = 6.86021 R-Sq = 98.23% R-Sq(adj) = 97.50% |  |  |  |  |  |  |

By Table 3.2, we find R-Sq(adj) = 97.5%, which means the model is good. The model explained 97.5% to the data, and p-value are lower than 0.05, so the simplified model is valid.

ANOVA model

From the ANOVA analysis, the model is presented as

Yijkl＝μ＋Ai＋Bj＋Ck＋Dl＋ABij＋BCjk＋CDkl＋ADil＋BDjl  ;  i=j=k=l=1,2

| Table 3.3 ANOVA model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Term | Effect | Coef | SE Coef | T | P |
| Constant |  | 47 | 1.213 | 38.76 | <0.001 |
| A | 35.13 | 17.56 | 1.213 | 14.48 | <0.001 |
| B | -35 | -17.5 | 1.213 | -14.43 | <0.001 |
| C | 57.5 | 28.75 | 1.213 | 23.71 | <0.001 |
| D | 26.62 | 13.31 | 1.213 | 10.98 | <0.001 |
| A*B | -15.12 | -7.56 | 1.213 | -6.24 | <0.001 |
| A*D | 10.25 | 5.12 | 1.213 | 4.23 | <0.001 |
| B*C | -13.75 | -6.87 | 1.213 | -5.67 | <0.001 |
| B*D | -7.12 | -3.56 | 1.213 | -2.94 | 0.008 |
| C*D | 10.88 | 5.44 | 1.213 | 4.48 | <0.001 |

By Table 3.3, we can write a model to do prediction. The model is

Yijkl＝47＋Ai＋Bj＋Ck＋Dl＋ABij＋BCjk＋CDkl＋ADil＋BDjl ;  i=j=k=l=1,2

and enter the parameter predicted by Table 3.4.

| Table 3.4 ANOVA Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| factor | level | Parameter | level | Parameter |
| A | -1 | -17.56 | 1 | 17.56 |
| B | -1 | 17.5 | 1 | -17.5 |
| C | -1 | -28.75 | 1 | 28.75 |
| D | -1 | -13.31 | 1 | 13.31 |
| A*B | -1 ,-1 | -7.56 | -1,+1 | 7.56 |
|  | +1,-1 | 7.56 | +1,+1 | -7.56 |
| A*D | -1 ,-1 | 5.12 | -1,+1 | -5.12 |
|  | +1,-1 | -5.12 | +1,+1 | 5.12 |
| B*C | -1 ,-1 | -6.87 | -1,+1 | 6.87 |
|  | +1,-1 | 6.87 | +1,+1 | -6.87 |
| B*D | -1 ,-1 | -3.56 | -1,+1 | 3.56 |
|  | +1,-1 | 3.56 | +1,+1 | -3.56 |
| C*D | -1 ,-1 | 5.44 | -1,+1 | -5.44 |
|  | +1,-1 | -5.44 | +1,+1 | 5.44 |

Models of appropriate verification:

| Figure 3.5 Residual against predicted. | By Figure 3.5, distribution has no special trends, it meets the residual variation of number-average value of the assumptions. |
| --- | --- |
| Figure 3.6 Normal Probability Plot. | By Figure 3.6, the points of data present attach to a straight line, so we assume that the data are under normal distribution. |
| Figure 3.7 Residual against Obs. Order. | By Figure 3.7, distribution isn't special trends, which mean that comply with the independence of the residual assumptions. |

Regression analysis

Following the ANOVA analysis, and placed the affective factors in the model. So we consider A, B, C, D, AB, BC, CD, AD and BD in the model and shown as show below: Y = 47.0 + 17.6 A - 17.5 B + 28.8 C + 13.3 D - 7.56 AB - 6.88 BC + 5.44 CD

+ 5.13 AD - 3.56 BD

CONCLUSION

We use the fractional factorial design (25-1) to make the experiment. When we consider main effects, the best combination is A(1)B(-1)C(1)D(1), which mean A in high level, B in low level, C in high level and D in high level. That means Upright Arm Tension Location in high level, Turn Table Position in low level, Pivot Arm Tension Location in high level and Ball Seat Position in high level. In this best combination, we can throw the ball to the longest distance. Because we don't use all factors, we may miss some effects. We do only two times to the experiment, so we don't have enough data to estimate interactions which are high. When we made the experiment, we use flour to avoid the error of human sight. But the CAT-100 catapult may move during the experiment time when we throw the ball. So we change the level and start to throw the ball, we have to be carefully and make the CAT-100 catapult remain the same position.

- REFERENCES

Montgomery, D. C. (2009). Statistical quality control (sixth ed.). U.S.A: John Wiley & Sons, Inc.

Pan, J.N, Lee, W.-J. Quality Management - Principles and Practices, 2nd (潘浙楠, & 李文瑞. (2003). 品質管理 (2 ed.). 台北: 華泰書局)

Montgomery, Douglas C.,”Design and Analysis of Experiments”